## Components Table

| Component | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| ESP32 Microcontroller | 1 per bot (2 total) | The central processing unit handles BLE communication, reads the touch sensor, runs game logic, and drives all outputs simultaneously | Dual-core architecture handles BLE and game loop concurrently; built-in BLE and capacitive touch hardware eliminates the need for separate modules |
| L298N Motor Driver | 1 per bot (2 total) | Translates ESP32 digital direction signals into the higher current needed to drive DC motors | L298N handles and supports bidirectional control via IN1/IN2 logic |
| DC Motor | 2 per bot (4 total) | direction between left and right motors, produces forward, backwards, | Two motors provide enough torque for pushing and shoving in sumo combat |
| Servo Motor | 2 per bot (4 total) | Actuate the attack arms rotate from 0° (neutral) to 180° (attack) on command | Precise angle control via PWM duty cycle allows repeatable arm positions |
| Capacitive Touch Film | 1 per bot (2 total) | Hit detection registers when the opponent bot makes physical contact with the body | Passive sensing with no moving parts; wraps around the bot body to detect contact from any angle; ESP32 TouchPad reads it natively without additional circuitry |
| NeoPixel LED Strip | 1 per bot (2 total) | Visual HP bar: each of 8 LEDs represents 1 HP; colour changes from green to orange to red as HP decreases | Single data wire controls all 8 LEDs independently via WS2812B protocol; RGB colour gives instant visual game state readout visible to spectators |
| Buzzer | 1 per bot (2 total) | Audio feedback: short beep on hit, 3-second continuous tone on defeat | Immediate auditory confirmation of hit events supplements visual LED feedback; PWM frequency control allows different tones for hit vs defeat |
| Battery Pack (7.4V LiPo) | 1 per bot (2 total) | Independent power supply for fully wireless, untethered operation during the match | Self-contained power eliminates tethering cables that would restrict movement and create entanglement hazards in combat; LiPo provides high current delivery for motor loads |

## Electronics Block Diagram

POWER FLOW

Battery Pack (7.4V LiPo)

│

├──→ L298N Motor Driver (Vin)

│         │

│         ├──→ Left DC Motor

│         └──→ Right DC Motor

│

├──→ ESP32 (via onboard 3.3V regulator or 5V USB pin)

│

└──→ NeoPixel Strip (5V) + Buzzer (3.3V via ESP32 pin)

SIGNAL FLOW

Mobile Phone

│

│  Bluetooth LE (BLE)

│  Commands: F / B / L / R / S / A (as bytes)

│  Receives: HP:8 → HP:0 (as string notification)

▼

ESP32 Microcontroller (Central Processing Unit)

│

├──→ Pin 22, 23 (Digital OUT) ──→ L298N IN1, IN2 ──→ Left DC Motor

├──→ Pin 19, 18 (Digital OUT) ──→ L298N IN3, IN4 ──→ Right DC Motor

│

├──→ Pin 16 (PWM 50Hz) ──→ Servo Motor 1 (Left Arm)

├──→ Pin 17 (PWM 50Hz) ──→ Servo Motor 2 (Right Arm)

│

├──→ Pin 32 (TouchPad IN) ──← Capacitive Touch Film (Hit Detection)

│

├──→ Pin 13 (Digital OUT) ──→ NeoPixel LED Strip (HP Bar)

│

└──→ Pin 25 (PWM OUT) ──→ Buzzer (Audio Feedback)