# Test Plan & Results

### 1. Test Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| BLE Pairing (Bot 1) | Open app, scan for “RoboSumo-1”, tap to connect | App shows “Connected” within 5 s; ESP32 prints “Connected” |
| BLE Pairing (Bot 2) | Open app, scan for “RoboSumo-2”, tap to connect | App shows “Connected” within 5 s; ESP32 prints “Connected” |
| Forward / Backward Drive | Send F then B commands from the app | Both wheels spin same direction; bot moves straight forward then backward |
| Left / Right Pivot | Send L then R commands | Left: left wheel reverses, right wheel forward. Right: opposite |
| Stop Command | Send S while bot is driving | Both motors stop immediately, bot halts |
| Servo Arm Toggle | Send command “1” twice, then “2” twice | Left arm extends ~180° then retracts ~0°; same for right arm |
| Capacitive Touch Hit Detection | Press opponent bot body against touch film manually | Hit registered, HP decrements by 1, buzzer beeps, one NeoPixel LED turns off |
| Hit Cooldown | Hold bots in continuous contact for 3 seconds | Only one hit per 500 ms window, not multiple instant hits |
| False Hit Rejection | Drive bot around arena for 60 s without any contact | Zero hits from vibration, motor noise, or stray capacitance |
| NeoPixel HP Display | Register hits from 8 HP down to 0 | Strip starts 8 green LEDs; each hit turns off one LED; count stays accurate |
| Death Sequence | Drain HP to 0 | Motors lock, strip goes red, buzzer sounds 3 s, strip goes dark, bot ignores commands |
| Battery Endurance | Fully charge battery, run continuous driving + periodic hits | Bot operates normally for at least 15 minutes without brownout |

### 2. User Testing

Who were the users?

Three classmates (ages 17–18) with no prior experience with the bot or the app. None had seen the code or wiring beforehand.

What tasks were given?

Task 1 — Connect: “Open the app and connect to the bot called RoboSumo-1.”

Task 2 — Drive: “Drive the bot forward, turn left, then stop.”

Task 3 — Arms: “Use the arm buttons to push something in front of the bot.”

Task 4 — Combat: “Play a 1v1 match against another player. Try to hit their bot.”

Task 5 — Read HP: “Without asking me, tell me how much health your bot has left.” (mid-match, after 3 hits)

### 3. Results

| Test | Outcome | Pass/Fail |
| --- | --- | --- |
| BLE Pairing (Bot 1) | App detected and connected on first tap within 2 seconds | Pass |
| BLE Pairing (Bot 2) | App detected the bot but connection failed repeatedly; ESP32 never printed “Connected”. Suspected faulty BLE antenna or firmware flash issue on second unit | Fail |
| Forward / Backward Drive | Both directions worked correctly; bot drove straight | Pass |
| Left / Right Pivot | Pivot steering worked; bot turned in correct direction | Pass |
| Stop Command | Bot stopped immediately on S command | Pass |
| Servo Arm Toggle | Both arms extended and retracted correctly on toggle | Pass |
| Capacitive Touch Hit Detection | Hits were detected, but not reliably when intended — direct front-on contact registered, but glancing and angled contact often missed. Some false positives when bot drove over rough surface | Fail |
| Hit Cooldown | Continuous contact registered hits at 500 ms intervals as designed | Pass |
| False Hit Rejection | Two false hits triggered during 60-second drive over uneven arena floor | Fail |
| NeoPixel HP Display | LED count matched HP accurately from 8 down to 0 | Pass |
| Death Sequence | Motors locked, strip went red, buzzer sounded ~3 s, strip went dark | Pass |
| Battery Endurance | Bot ran for ~20 minutes before voltage sag caused motor stuttering | Pass |

### 4. Observations

User Behaviour

All three users understood the D-pad layout immediately — forward/back/left/right mapping required no explanation. Users instinctively held the drive buttons (TouchDown behaviour) rather than tapping, which matches the app design. During combat, players focused on the physical arena rather than the phone screen; the NeoPixel strip was their primary HP feedback. Two users tried pressing forward and left simultaneously to curve, but the app only sends one command at a time, so the bot pivoted instead of curving.

Pain Points

Capacitive touch unreliability: The biggest issue. Hits registered on solid front-on contact, but glancing or angled contact was missed roughly 40% of the time. The threshold that avoided false positives during solo driving was too conservative for reliable hit detection during actual combat. Additionally, two false positives occurred when the bot drove over bumps in the arena surface.

Bot 2 BLE failure: The second bot’s Bluetooth connection failed entirely — the phone detected it during scan but could never establish a stable connection. The ESP32 never printed “Connected” in the serial monitor. This may be a hardware issue (faulty antenna or damaged ESP32 module) or a firmware flash problem. Testing proceeded using Bot 1 only, with users taking turns.

No diagonal movement: Players expected joystick-style analogue control (forward + left = gentle curve). The D-pad only supports one command at a time, making movement feel rigid.

No match reset without power cycle: After a bot died, players had to physically turn it off and on. They expected a “rematch” button on the app.

### 6. Improvements Suggested

Based on testing:

1. Replace capacitive touch film with mechanical bump switches (microswitches). This was the most critical failure. Microswitches on the front and sides of the bot would give reliable, binary hit detection regardless of contact angle, eliminating both false negatives (missed hits) and false positives (vibration triggers). No threshold tuning required.

2. Debug and fix Bot 2 BLE. Re-flash the firmware using a known-good USB cable; if the issue persists, swap the ESP32 module. Test with nRF Connect to isolate whether the problem is hardware or software.

3. Add a soft-reset BLE command. A new command (e.g. “X”) that resets HP to 8, repaints the NeoPixel strip, clears the match-over flag, and re-enables motors — removing the need to power-cycle between matches.

4. Replace D-pad with a joystick Canvas in MIT App Inventor. Map the X/Y touch position to independent left/right motor speeds, enabling curved movement and variable speed. This was the most requested improvement from user testing.

5. Add BLE connection confirmation on the bot. Flash the NeoPixel strip blue for 1 second when a phone connects, giving the player physical confirmation that the link is established.

6. Use a larger battery. A 2S LiPo (2200 mAh) would extend match time beyond 20 minutes and reduce voltage sag that caused motor stuttering near the end of the endurance test.